Note to users: items shown in red indicated edits / selections that need to be made to tailor the specification for the application.

Grooved End Center Guided Silent Check Valve.

- General:

  - Grooved end center guided check valves shall be installed where shown on plans or required by equipment manufacturers.

  - Check valve shall be certified to be in conformance with NSF 372.

- Product:

  - Grooved End Center Guided Silent Check Valve shall be model “CVO 700" as manufactured by The Metraflex Company®, Chicago, IL.

  - Materials:

    - Check valve body shall be ASTM A126 B Cast Iron.

    - Check valve trim.

      - Check valve trim shall be nickel coated ductile iron (recommended).

      - Check valve trim shall be Type 304 Stainless steel.

      - Check valve trim shall be Type 304 Stainless steel with buna seal.

  - Globe style center guided check valves shall be comply to MSS-SP-125-2010.

  - Grooved ends shall made from ASTM A-395 ductile iron.

- Execution:

  - Grooved end center guided check valves shall be installed in accordance with manufacturer’s recommendations and contact requirements.

Product Specifications