Lug Style Butterfly Valve.

- General:

  - Lug style butterfly valves shall be installed where shown on plans or required by equipment manufacturers.

  - Butterfly valve shall be certified to be in conformance with NSF 372.

- Product:

  - Lug style butterfly valve shall be model “BL" as manufactured by The Metraflex Company®, Chicago, IL.

  - Materials:

    - Butterfly valve body shall be ASTM A126 B Cast Iron.

    - Butterfly disk shall be nickel coated ductile iron in accordance with ASTM D 536.

    - Liner shall be EPDM in accordance with ASTM D-2000

    - Stem shall be AISI 420 stainless steel.

- Execution:

  - Butterfly valves shall be installed in accordance with manufacturer’s recommendations and contact requirements.

  - Check valve shall be installed between ASME B16.1 flanges.

  - Installer shall insure there is no interference butterfly valve and adjacent equipment.

Product Specifications