Bellows Connector – Expansion Joint.  
Style MPTR

www.Metraflex.com

- General:

  - Flexible metal bellows connectors shall be installed where indicated on drawings.

  - Flexible metal bellows connectors/expansion joints shall be of the internally pressurized multiple ply bellows type with floating flanges.

  - Flexible metal bellows connectors are suitable for axial and lateral movement

- Product:

  - Flexible bellows pump connectors/expansion joints shall be model “MPTR" as manufactured by The Metraflex Company®, Chicago, IL.

  - Bellows shall be minimum two ply T304 Stainless steel.

  - Flanges shall be Van Stone style 150 lb. floating carbon steel.

  - Flanges shall have integral control rods installed with rubber grommets to prevent overextension.

  - All wetted surfaces shall be Type 304 stainless steel.

  - Connector Pressure Rating

    - 150°F Maximum Working Pressure: 225 psi

    - 212°F Maximum Working Pressure: 190 psi

    - 480°F Maximum Working Pressure: 110 psi

- Execution:

  - Consult the factory submittals for specific performance specifications of joint.