Note to users: items shown in red indicated edits / selections that need to be made to tailor the specification for the application.

Product Specifications

Slide Type Pipe Guides.

- General

  - Provide pipe guides as indicated on project plans or in accordance with EJMA guidelines based on design pressure and line size.

  - Primary and intermediate guides shall be slide type.

- Products

  - Pipe guides shall be style “PTFE Slide Guide” as manufactured by The Metraflex Company®, Chicago, IL.

  - All components shall be manufactured in compliance with the Manufacturer's Standardization Society (MSS) SP-58 and SP-89

  - Guide shall be made up of a base plate and slide assembly. Base plate shall be;

    - Bolt down

    - Weld to structure.

    - Slide type

  - Guide shall be manufactured from ASTM A-134 carbon steel.

  - Guide slide bearing pad shall be PTFE.

  - Slide assembly shall be sized to accommodate pipe insulation.

  - All surfaces of guide shall be coated with fusion bonded powder coating.

- Execution

  - Install and guide per manufacturer’s installation instructions and Mechanical Contractors Association of America “Guidelines for Quality Piping Installations”.