Split Disk Wafer Style Check Valve.

- General:

  - Split disk style check valves shall be installed where shown on plans or required by equipment manufacturers.

  - Check valve shall be certified to be in conformance with NSF 372.

- Product:

  - Wafer Style Center Guided Silent Check Valve shall be model “CVXXK" as manufactured by The Metraflex Company®, Chicago, IL.

  - Materials:

    - Check valve body shall be ASTM A126 B Cast Iron with fusion bonded epoxy coating.

    - Check valve disk shall be CF8 stainless steel.

    - Check valve spring shall be Type 304 Stainless steel.

.

- Execution:

  - Split disk wafer style check valves shall be installed in accordance with manufacturer’s recommendations and contact requirements.

  - Check valve shall be installed between ASME B16.1 flanges.

  - Installer shall insure there is no interference between check valve and adjacent equipment.

Product Specifications